1990 CENSUS TRANSPORTATION PLANNING PACKAGE

FILE FORMAT FOR THE FINAL BLOCK-TO-TAZ EQUIVALENCY FILE

TAZ.DOC FILE

The census block-to-traffic analysis zone equivalency file

used by the Bureau of the Census in producing data for

the 1990 Census Transportation Planning Package (CTPP)

contains one record for each block in a county. Each record

has the following structure:

Characters  1 -  2   the FIPS State code for the block

Characters  3 -  5   the FIPS county code for the block

Characters  6 -  9   the four-digit census basic tract code

(right-justified and filled with

leading zeros)

Characters 10 - 11   the two-digit census tract suffix code

(blank if no suffix is present)

Characters 12 - 14   the three-digit census block code

Character  15        the one-character alphabetic block suffix

code (blank if no suffix is present)

Characters 16 - 18   the census 3-digit minor civil division

code (MCD)

Characters 19 - 22   the census 4-digit place code (blank if

the block is not located within a place)

Characters 23 - 28   the six-character TAZ code (right-justified

and filled with leading blanks)

TAZ codes may be any combination of up to six numbers and/or

alphabetic characters. Blank spaces are not permitted

between consecutive characters of a TAZ code, nor are special

characters allowed (-, /, *, &, etc.). If less than six

characters are used, the TAZ code is right-justified in the

field and filled with leading blanks.

TAZs were designated for entire counties (except in the

six New England States where whole MCDs or towns were

covered). If an MPO's study area encompassed part of a county

but not all of it,  additional TAZs were designated

to cover the territory outside the study area boundaries

but inside the county. We call these additional zones

"dummy zones". Dummy TAZs may have been given codes outside

the range of the real zone numbers. For example, if the zones

within the study area were numbered from 1 to 500, the dummy

zones might begin with the number 700.

When assigning census blocks to Traffic Analysis Zones, blocks

with the same block code but a different block suffix are

treated as separate blocks. Therefore, blocks with different

suffixes may be assigned to different TAZs. For example, block

315A could be assigned to TAZ 32 and block 315B to TAZ 33.

Other blocks, however, are not allowed to be split between TAZs.

Water blocks are those whose block code is x99, where x may

be any number. In most cases the MPOs were not asked to provide

TAZ codes for water blocks, and in the block-to-TAZ equivalency

files most water blocks have been assigned a TAZ code of "WATER".

This was done because in the 1990 Census Tiger Line files the

water bodies were all given the same census block code. In general

this was not a problem in the census, since by definition no

residences were located in these areas. Some workers' workplaces,

however, were legitimately coded to water blocks.